Technical Requirements

Main site

Virtualization Cluster

- Virtualization solution should be based on VMware vSphere.

- Solution should be based on a clustered architecture.

- The solution should be based on x86 architecture. Requirements –

CPU – Intel Gold 6300 series or better. Total physical cores–366 (2:1). Minimum number of servers is 4.

RAM – 2.3 TB total.

Network – 1 x 2-port 10/25Gb SFP28 NIC per node.

FC – 1 x 2-port 32 Gb/s adapter per node.

Power – Redundant power supplies.

Boot device – 2 x 480 M.2 SSD.

Licensing – All required licenses (vSphere Enterprise Plus, etc.) to be included. No vCenter license required. Licensing should be based on perpetual model. No subscription licensing model to be used.

Hypervisor – VMware vSphere.

Cabling – All required components (transceivers, DAC cables) to be included. Two 25 Gb/s 10 meters AOC cables per node should be included for public connection.

Reliability – Running services should be protected by hardware and software high availability features.

Maintenance - non-disruptive upgrades of firmware and software.

Support – 3 years 24/7, 4 hours response time. Local spares stock from vendor.

Remote monitoring – Automatic reporting of hardware failures to manufacturer’s support center.

Installation - Physical installation, cabling, labeling, initial startup, configuration and firmware update.

Shared Disk Array

- Disk array should be based on SSD (All Flash) technology. Requirements –

Architecture – Active/Active Controllers.

Space – SAS SSD at least 20 TB usable after RAID 6, upgradeable to at least 80 TB (without adding  enclousers or additional storage). No data reduction technologies to be used.

Storage performance – IOPS at least 70000, <60% utilization with latency less than 1 ms. Usage pattern – 60% read/40% write, 8K random.

External ports – 4, 16 Gb/s speed minimum.

Redundancy – Dedicated or distributed hot spare.

Reliability – Redundant hardware components.

Support – 3 years 24/7, 4 hours response time. Local spares stock from vendor.

Remote monitoring – Automatic reporting of hardware failures to manufacturer’s support center.

Installation - Physical installation, cabling, labeling, initial startup, configuration and firmware update.

Disaster Recovery site

Virtualization Cluster

- Virtualization solution should be based on VMware vSphere.

- Solution should be based on a clustered architecture.

- The solution should be based on x86 architecture. Requirements –

CPU – Intel Gold 6300 series or better. Total physical cores – 140. Minimum number of servers is 3.

RAM – 512 GB total.

Network – 1 x 2-port 10/25Gb SFP28 NIC per node.

FC – 1 x 2-port 32 Gb/s adapter per node.

Power – Redundant power supplies.

Boot device – 2 x 480 M.2 SSD.

Licensing – All required licenses (vSphere Enterprise Plus, etc.) to be included. No vCenter license required. Licensing should be based on perpetual model. No subscription licensing model to be used.

Hypervisor – VMware vSphere.

Cabling – All required components (transceivers, DAC cables) to be included. Two 25 Gb/s 10 meters AOC cables per node should be included for public connection.

Reliability – Running services should be protected by hardware and software high availability features.

Maintenance - non-disruptive upgrades of firmware and software.

Support – 3 years 24/7, 4 hours response time. Local spares stock from vendor.

Remote monitoring – Automatic reporting of hardware failures to manufacturer’s support center.

Installation - Physical installation, cabling, labeling, initial startup, configuration and firmware update.

Shared Disk Array

- Disk array should be based on SSD (All Flash) technology. Requirements –

Space – SAS SSD at least 6 TB usable after RAID 6, upgradeable to at least 80 TB (without adding enclousers or additional storage). No data reduction technologies to be used.

Storage performance – IOPS at least 50000,  <60% utilization with latency less than 1 ms. Usage pattern – 60% read/40% write, 8K random.

External ports – 4, 16 Gb/s speed minimum.

Redundancy – Dedicated or distributed hot spare.

Reliability – Redundant hardware components.

Support – 3 years 24/7, 4 hours response time. Local spares stock from vendor.

Remote monitoring – Automatic reporting of hardware failures to manufacturer’s support center.

Installation - Physical installation, cabling, labeling, initial startup, configuration and firmware update.

General Requirements

·        İştirakçı bu layihə üçün təqdim etdiyi məhsullar üzrə İstehsalçının Avtorizasiya Formasını (MAF) təqdim etməlidir;

·        İştirakçı Azərbaycan Respublikasında qeydiyyatdan keçmiş hüquqi şəxs və ya fərdi sahibkar olmalıdır;

·        İştirakçı Xidmət üzrə ən azı 3 (üç) il təcrübəyə malik olmalı və təklif olunan həll və xidmət üzrə ən azı 3 (üç) müştərisi və/və ya layihəsi olmalıdır;

·        İştirakçı tələbləri özü icra etməli və üçüncü şəxslərə həvalə etməməlidir;

·        İştirakçı Tender Şərtləri toplusunda olan, forma, şərtlər, spesifikasiyalar və digər bütün məlumatlarla ətraflı tanış olmalıdır. Qeyri-dəqiq və ya tamam olmayan tender təklifi vermək İştirakçının öz məsuliyyətindədir və bu təklifin qəbul edilməməsinə səbəb ola bilər;

·        İştirakçı elan olunmuş istiqamətdə təsdiq edici sertifikatlara malik olmalıdır.

.        İştirakçı təqdim etdiyi avadanlığın perfomans göstəriciləri haqqında zəmanət məktubu (sənədi) təqdim etməlidir.

.        Iştirakçı avadanlığın perfomans göstəriciləri haqqında, vendor/istehsalçı tərəfindən təmin edilmiş rəsmi zəmanət məktubu (sənədi) təqdim etməlidir.

.        Iştirakçı, vendor tərəfindən təsdiqlənmiş 3 illik TCO (ümumi mülkiyyət dəyəri) sənədini təqdim etməlidir.